Aim :

| Ex. No : 1 | STUDY OF MATLAB | Date : |
| --- | --- | --- |

Result :

Aim :

| Ex. No : 2 | GENERATION OF SIGNALS | Date : |
| --- | --- | --- |

Program :

%program to generate unit impulse signal

clc;

clear all;

close all;

t=-2:1:2;

y=[zeros(1,2),ones(1),zeros(1,2)];

figure;

stem(t,y);

ylabel('Amplitude--------->');

xlabel('Time------------>');

title('Unit Impulse Signal');

Output :

Program :

%Program to generate unit step signal(discrete signal);

clc;

close all;

clear all;

n=input('Enter the length of the sequence');

t=0:1:n-1;

y=[ones(1,n)];

stem(t,y);

ylabel('Amplitude------>');

xlabel('Time------>');

title('Unit step signal');

Output :

Enter the length of the sequence10

>>

Program :

%Program to generate the unit step signal (continuous signal) ;

clc;

clear all;

close all;

n=input('Enter the length of the sequence');

t=0:1:n-1;

y=[ones(1,n)];

plot(t,y);

ylabel('Amplitude---------->');

xlabel('Time-------->');

title('Unit step signal');

Output :

Enter the length of the sequence10

>>

Program :

%Program to generate unit ramp signal(discrete signal);

clc;

clear all;

close all;

n=input('Enter the length of the sequence');

t=0:1:n-1;

stem(t,t);

ylabel('Amplitude------>');

xlabel('Time-------->');

title('Unit ramp signal');

Output :

Enter the length of the sequence10

>>

Program :

%Program to generate unit ramp signal(continuous signal);

clc;

clear all;

close all;

n=input('Enter the length of the sequence');

t=0:1:n-1;

plot(t,t);

ylabel('Amplitude----->');

xlabel('Time---->');

title('Unit ramp signal');

Output :

Enter the length of the sequence10

>>

Program :

%Program to generate sinusoidal signal;

clc;

clear all;

close all;

f=input('Enter the frequency of the signal');

t=0:0.01:pi;

y=sin(2*pi*f*t);

plot(t,y);

ylabel('Amplitude-------->');

xlabel('Time--------->');

title('Sinusoidal signal');

Output :

Enter the frequency of the signal1

>>

Program :

%Program to generate co sinusoidal signal;

clc;

clear all;

close all;

f=input('Enter the frequency of the signal');

t=0:0.01:pi;

y=cos (2*pi*f*t);

plot(t,y);

ylabel('Amplitude-------->');

xlabel('Time----->');

title('Co sinusoidal signal');

Output :

Enter the frequency of the signal1

>>

Program :

%Program to generate discrete time signal;

clc;

clear all;

close all;

x=input('Enter the sequence');

figure;

stem(x);

ylabel('Amplitude-------->');

xlabel('Time--------->');

title('Discrete time signal');

Output :

Enter the sequence[1 3 -5 2]

>>

Result :

Aim :

Algorithm :

| Ex. No : 3 | IMPLEMENTATION OF FFT | Date : |
| --- | --- | --- |

Program:

%Program to calculate FFT;

clc;

clear all;

close all;

x=input('Enter the input sequence');

y=fft(x);

subplot(2,1,1);

stem(x);

xlabel('Time------->');

ylabel('Amplitude----------->');

subplot(2,1,2);

stem(y);

xlabel('Time-------->');

ylabel('Amplitude---------->');

disp('Input sequence');x

disp('FFT values');y

title('Calculation of FFT');

Output:

Enter the input sequence[1 2 3 4 4 3 2 1]

Input sequence

x =

1     2     3     4     4     3     2     1

FFT values

y =

Columns 1 through 6

20.0000            -5.8284 - 2.4142i        0            -0.1716 - 0.4142i        0            -0.1716 + 0.4142i

Columns 7 through 8

0            -5.8284 + 2.4142i

>>

Result:

| Ex. No : 4 | IMPLEMENTATION OF DFT | Date : |
| --- | --- | --- |

Aim :

Algorithm :

Program :

%Program to calculate DFT

clc;

clear all;

close all;

x=input('Enter the sequence');

N=length(x);

for k=0:1:N-1;

for n=0:1:N-1;

p=exp(-i*2*pi*k*n/N);

y(n+1,k+1)=p;

end;

end;

y=x*y;

disp(y);

subplot(2,1,1);

stem(x);

xlabel('Time-------->');

ylabel('Amplitude---------->');

title('Input sequence');

subplot(2,1,2);

stem(y);

xlabel('Time-------->');

ylabel('Amplitude---------->');

title('DFT sequence');

Output :

Enter the sequence[1 2 3 4]

10.0000            -2.0000 + 2.0000i  -2.0000 - 0.0000i  -2.0000 - 2.0000i

>>

Result :

Aim :

Algorithm :

| Ex. No : 5 | LINEAR & CIRCULAR CONVOLUTION | Date : |
| --- | --- | --- |

Program :

%Program to calculate linear convolution;

clc;

clear all;

close all;

x1=input('Enter the input sequence');

x2=input('Enter the input sequence');

y=convn(x1,x2);

subplot(3,1,1);

stem(x1);

ylabel('Amplitude-------->');

xlabel('Time---------->');

title('Input sequence x1');

subplot(3,1,2);

stem(x2);

ylabel('Amplitude-------->');

xlabel('Time---------->');

title('Input sequence x2');

subplot(3,1,3);

stem(y);

ylabel('Amplitude-------->');

xlabel('Time---------->');

disp('Input sequence x1');

disp(x1);

disp('Input sequence x2');

disp(x2);

disp('Convn value y');

disp(y);

title('Convolution');

Output :

Enter the input sequence[0 1 2 3]

Enter the input sequence[1 2 3 4]

Input sequence x1

0     1     2     3

Input sequence x2

1     2     3     4

Convn value y

0     1     4    10    16    17    12

>>

Program :

%Program to calculate circular convolution;

clc;

clear all;

close all;

x=input('Enter the input sequence(x)');

h= input('Enter the input sequence(h)');

y=fft(x);

n=fft(h);

for i=1:length(x)

s(i)=y(i)*n(i);

end

z=ifft(s);

subplot(3,1,1);

stem(x);

ylabel('Amplitude-------->');

xlabel('Time---------->');

title('x(n)');

subplot(3,1,2);

stem(h);

ylabel('Amplitude-------->');

xlabel('Time---------->');

title('h(n)');

subplot(3,1,3);

stem(z);

ylabel('Amplitude-------->');

xlabel('Time---------->');

disp('Input sequence x(n)');

disp(x);

disp('Input sequence');

disp(h);

disp('Convn values');

disp(z);

title('Circular Convolution');

Output :

Enter the input sequence(x)[0 1 2 3]

Enter the input sequence(h)[1 2 3 4]

Input sequence x(n)

0     1     2     3

Input sequence

1     2     3     4

Convn values

16    18    16    10

>>

Result :

Aim :

Algorithm :

| Ex. No : 6 | SAMPLING THEOREM | Date : |
| --- | --- | --- |

Program:

%Solution for section(a);

clc;

clear all;

close all;

f1=1/128;

f2=5/128;

n=0:255;

fc=50/128;

x=cos(2*pi*f1*n)+cos(2*pi*f2*n);

xa=cos(2*pi*fc*n);

xamp=x.*xa;

subplot(2,2,1);

plot(n,x);

title('x(n)');

xlabel('n------>');

ylabel('Amplitude--------?');

subplot(2,2,2);

plot(n,xa);

title('xa(n)');

xlabel('n------>');

ylabel('Amplitude--------?');

subplot(2,2,3);

plot(n,xamp);

xlabel('n-------->');

ylabel('Amplitude--------->');

%128 point DFT computation-solution for section(b);

n=0:127;

figure;

n1=128;

f1=1/128;

f2=5/128;

fc=50/128;

x=cos(2*pi*f1*n)+ cos(2*pi*f2*n);

xc= cos(2*pi*fc*n);

xamp=x.*xc;

xam=fft(xamp,n1);

stem(n,xam);

title('xamp(n)');

xlabel('n-------->');

ylabel('Amplitude--------->');

%128 point DFT computation-solution for section(c);

n=0:99;

figure;

n2=0:n1-1;

f1=1/128;

f2=5/128;

fc=50/128;

x=cos(2*pi*f1*n)+ cos(2*pi*f2*n);

xc= cos(2*pi*fc*n);

xamp=x.*xc;

for i=1:100

xamp1(i)=xamp(i);

end

xam=fft(xamp1,n1);

stem(n2,xam);

title('xamp(n)');

xlabel('n-------->');

ylabel('Amplitude--------->');

Output:

Result :

| Ex. No : 7 | INTERPOLATION | Date : |
| --- | --- | --- |

Aim :

Algorithm :

Program :

%Program for interpolation

clc;

clear all;

close all;

N=input('input length=');

L=input('up-sampling factor=');

f0=input('input signal frequency=');

%generate the input sinusoidal sequence%

n=0:N-1;

x=sin(2*pi*f0*n);

%generate the up sampled sequence%

y=zeros (1,L*length(x));

y([1:L:length(y)])=x;

%to plot the input and output sequences

subplot(2,1,1);

stem(n,x);

xlabel('time index n');

ylabel('amplitude');

title('input sequence');

subplot(2,1,2);

stem(n,y (1:length(x)));

xlabel('time index n');

ylabel('amplitude');

title(['output sequence up-sampled by:num2str(L)']);

Output :

input length=50

up -sampling factor=3

input signal frequency=.2

>>

Result :

Aim :

Algorithm :

| Ex. No : 8 | DECIMATION | Date : |
| --- | --- | --- |

Program :

%Program for decimation

clc;

close all;

clear all;

N=input('output length=');

M=input('down sampling factor=');

f0=input('input signal frequency=');

%generate the input sinusoidal sequence%

n=0 : N-1;

m=0 : N*M-1;

x=sin(2*pi*f0*m);

%generate the down sampled sequence%

y=x([1:M:length(x)]);

%to plot the input and output sequence%

subplot(2,1,1);

stem(n,x(1:N));

title('input sequence');

subplot(2,1,2);

stem(n,y);

title(['output sequence down sampled by:num2str(m)']);

xlabel('time index');

ylabel('amplitude');

Output :

output length=50

down sampling factor=3

input signal frequency=.2

>>

Result :

| Ex. No : 9 | FIR FILTER DESIGN-RECTANGULAR WINDOW | Date : |
| --- | --- | --- |

Aim :

Algorithm :

Program :

%Program for design FIR lowpass, bandpass, band stop filter using rectangular window;

clc;

clear all;

close all;

rp=input('Enter the pass band ripple');

rs=input('Enter the stop band ripple');

fp=input('Enter the pass band frequency');

fs=input('Enter the stop band frequency');

f=input('Enter the sampling  frequency');

wp=2*fp/f;

ws=2*fs/f;

num=-20*log10(sqrt(rp*rs))-13;

dem=14.6*(fs-fp)/f;

n=ceil(num/dem);

n1=n+1;

if(rem(n,2)~=0);

n1=n;

n=n-1;

end

y=boxcar(n1);

%low pass filter;

b=fir1(n,wp,y);

[h,o]=freqz(b,1,256);

m=20*log10(abs(h));

subplot(2,2,1);

plot(o/pi,m);

xlabel('(a)Normalized frequency------------->');

ylabel('Gain in dB--------->');

%High pass filter;

b=fir1(n,wp,'high',y);

[h,o]=freqz(b,1,256);

m=20*log10(abs(h));

subplot(2,2,2);

plot(o/pi,m);

xlabel('(b)Normalized frequency------------->');

ylabel('Gain in dB--------->');

%Band pass filter;

wn=[wp,ws];

b=fir1(n,wn,y);

[h,o]=freqz(b,1,256);

m=20*log10(abs(h));

subplot(2,2,3);

plot(o/pi,m);

xlabel('(c)Normalized frequency------------->');

ylabel('Gain in dB--------->');

%Band stop filter;

b=fir1(n,wn,'stop',y);

[h,o]=freqz(b,1,256);

m=20*log10(abs(h));

subplot(2,2,4);

plot(o/pi,m);

xlabel('(d)Normalized frequency------------->');

ylabel('Gain in dB--------->');

Output :

Enter the pass band ripple0.05

Enter the stop band ripple0.04

Enter the pass band frequency1500

Enter the stop band frequency2000

Enter the sampling  frequency9000

>>

Result :

| Ex. No : 10 | FIR FILTER DESIGN-HAMMING WINDOW | Date : |
| --- | --- | --- |

Aim :

Algorithm :

Program :

%Design of FIR filter using Hamming window;

clc;

clear all;

close all;

rp=input('Enter the pass band ripple');

rs=input('Enter the stop band ripple');

fp=input('Enter the pass band frequency');

fs=input('Enter thestop band frequency');

f=input('Enter the sampling  frequency');

wp=2*fp/f;

ws=2*fs/f;

num=-20*log10(sqrt(rp*rs))-13;

dem=14.6*(fs-fp)/f;

n=ceil(num/dem);

n1=n+1;

if(rem(n,2)~=0);

n1=n;

n=n-1;

end

y=hamming(n1);

%low pass filter;

b=fir1(n,wp,y);

[h,o]=freqz(b,1,256);

m=20*log10(abs(h));

subplot(2,2,1);

plot(o/pi,m);

xlabel('(a)Normalized frequency------------->');

ylabel('Gain in dB--------->');

%High pass filter;

b=fir1(n,wp,'high',y);

[h,o]=freqz(b,1,256);

m=20*log10(abs(h));

subplot(2,2,2);

plot(o/pi,m);

xlabel('(b)Normalized frequency------------->');

ylabel('Gain in dB--------->');

%Band pass filter;

wn=[wp,ws];

b=fir1(n,wn,y);

[h,o]=freqz(b,1,256);

m=20*log10(abs(h));

subplot(2,2,3);

plot(o/pi,m);

xlabel('(c)Normalized frequency------------->');

ylabel('Gain in dB--------->');

%Band stop filter;

b=fir1(n,wn,'stop',y);

[h,o]=freqz(b,1,256);

m=20*log10(abs(h));

subplot(2,2,4);

plot(o/pi,m);

xlabel('(d)Normalized frequency------------->');

ylabel('Gain in dB--------->');

Output :

Enter the pass band ripple0.02

Enter the stop band ripple0.01

Enter the pass band frequency1200

Enter thestop band frequency1700

Enter the sampling  frequency9000

>>

Result :

| Ex. No : 11 | FIR FILTER DESIGN-HANNING WINDOW | Date : |
| --- | --- | --- |

Aim :

Algorithm :

Program :

%Design of FIR filter using Hanning window;

clc;

clear all;

close all;

rp=input('Enter the pass band ripple');

rs=input('Enter the stop band ripple');

fp=input('Enter the pass band frequency');

fs=input('Enter the stop band frequency');

f=input('Enter the sampling  frequency');

wp=2*fp/f;

ws=2*fs/f;

num=-20*log10(sqrt(rp*rs))-13;

dem=14.6*(fs-fp)/f;

n=ceil(num/dem);

n1=n+1;

if(rem(n,2)~=0);

n1=n;

n=n-1;

end

y=hanning(n1);

%low pass filter;

b=fir1(n,wp,y);

[h,o]=freqz(b,1,256);

m=20*log10(abs(h));

subplot(2,2,1);

plot(o/pi,m);

xlabel('(a)Normalized frequency------------->');

ylabel('Gain in dB--------->');

%High pass filter;

b=fir1(n,wp,'high',y);

[h,o]=freqz(b,1,256);

m=20*log10(abs(h));

subplot(2,2,2);

plot(o/pi,m);

xlabel('(b)Normalized frequency------------->');

ylabel('Gain in dB--------->');

%Band pass filter;

wn=[wp,ws];

b=fir1(n,wn,y);

[h,o]=freqz(b,1,256);

m=20*log10(abs(h));

subplot(2,2,3);

plot(o/pi,m);

xlabel('(c)Normalized frequency------------->');

ylabel('Gain in dB--------->');

%Band stop filter;

b=fir1(n,wn,'stop',y);

[h,o]=freqz(b,1,256);

m=20*log10(abs(h));

subplot(2,2,4);

plot(o/pi,m);

xlabel('(d)Normalized frequency------------->');

ylabel('Gain in dB--------->');

Output:

Enter the pass band ripple0.03

Enter the stop band ripple0.01

Enter the pass band frequency1400

Enter the stop band frequency2000

Enter the sampling  frequency8000

>>

Result :

Aim :

Algorithm :

| Ex. No : 12 | BUTTERWORTH DIGITAL IIR FILTERS | Date : |
| --- | --- | --- |

Program :

%Program for the design of Butterworth low pass filter;

clc;

clear all;

close all;

format long

rp=input('Enter the pass band ripple');

rs=input('Enter the stop band ripple');

fp=input('Enter the pass band frequency');

fs=input('Enter the stop band frequency');

f=input('Enter the sampling  frequency');

wp=2*fp/f;

ws=2*fs/f;

[n,wn]=buttord(wp,ws,rp,rs);

[b,a]=butter(n,wn);

w=0:.01:pi;

[h,om]=freqz(b,a,w);

m=20*log10(abs(h));

an=angle(h);

subplot(2,1,1);

plot(om/pi,m);

xlabel('(a)Normalized frequency------------->');

ylabel('Gain in dB--------->');

subplot(2,1,2);

plot(om/pi,an);

xlabel('(b)Normalized frequency------------->');

ylabel('Phase in radians--------->');

Output :

Enter the pass band ripple0.5

Enter the stop band ripple50

Enter the pass band frequency1200

Enter the stop band frequency2400

Enter the sampling  frequency10000

>>

Program :

%Program for design Butterworth high pass filter;

clc;

clear all;

close all;

format long

rp=input('Enter the pass band ripple');

rs=input('Enter the stop band ripple');

fp=input('Enter the pass band frequency');

fs=input('Enter the stop band frequency');

f=input('Enter the sampling  frequency');

wp=2*fp/f;

ws=2*fs/f;

[n,wn]=buttord(wp,ws,rp,rs);

[b,a]=butter(n,wn,'high');

w=0:.01:pi;

[h,om]=freqz(b,a,w);

m=20*log10(abs(h));

an=angle(h);

subplot(2,1,1);

plot(om/pi,m);

xlabel('(a)Normalized frequency------------->');

ylabel('Gain in dB--------->');

subplot(2,1,2);

plot(om/pi,an);

xlabel('(b)Normalized frequency------------->');

ylabel('Phase in radians--------->');

Output :

Enter the pass band ripple0.5

Enter the stop band ripple50

Enter the pass band frequency1200

Enter the stop band frequency2400

Enter the sampling  frequency10000

>>

Program :

%Program for design Butterworth band pass digital filter;

clc;

clear all;

close all;

format long

rp=input('Enter the pass band ripple');

rs=input('Enter the stop band ripple');

fp=input('Enter the pass band frequency');

fs=input('Enter thestop band frequency');

f=input('Enter the sampling  frequency');

wp=2*fp/f;

ws=2*fs/f;

[n]=buttord(wp,ws,rp,rs);

[wn]=[wp ws];

[b,a]=butter(n,wn,'bandpass');

w=0:.01:pi;

[h,om]=freqz(b,a,w);

m=20*log10(abs(h));

an=angle(h);

subplot(2,1,1);

plot(om/pi,m);

xlabel('(a)Normalized frequency------------->');

ylabel('Gain in dB--------->');

subplot(2,1,2);

plot(om/pi,an);

xlabel('(b)Normalized frequency------------->');

ylabel('Phase in radians--------->');

Output :

Enter the pass band ripple0.3

Enter the stop band ripple40

Enter the pass band frequency1500

Enter thestop band frequency2000

Enter the sampling  frequency9000

>>

Program :

%Program for design Butterworth band stop digital filter;

clc;

clear all;

close all;

format long

rp=input('Enter the pass band ripple');

rs=input('Enter the stop band ripple');

fp=input('Enter the pass band frequency');

fs=input('Enter the stop band frequency');

f=input('Enter the sampling  frequency');

wp=2*fp/f;

ws=2*fs/f;

[n]=buttord(wp,ws,rp,rs);

[wn]=[wp ws];

[b,a]=butter(n,wn,'stop');

w=0:.01:pi;

[h,om]=freqz(b,a,w);

m=20*log10(abs(h));

an=angle(h);

subplot(2,1,1);

plot(om/pi,m);

xlabel('(a)Normalized frequency------------->');

ylabel('Gain in dB--------->');

subplot(2,1,2);

plot(om/pi,an);

xlabel('(b)Normalized frequency------------->');

ylabel('Phase in radians--------->');

Output :

Enter the pass band ripple0.4

Enter the stop band ripple46

Enter the pass band frequency1100

Enter the stop band frequency2200

Enter the sampling  frequency6000

>>

Result :

| Ex. No : 13 | INTRODUCTION TO DSP PROCESSOR | Date : |
| --- | --- | --- |

Aim :

Result :

| Ex. No : 14 | FIR FILTER USING DSP PROCESSOR | Date : |
| --- | --- | --- |

Aim :

Algorithm :

Result :

| Ex. No : 15 | STUDY OF VARIOUS ADDRESSING MODES OF DSP | Date : |
| --- | --- | --- |

Result :